RG-RSR20-XA Series

Multi-service Access Routers

Datasheet

V1.0

Ruijie Networks Co., Ltd

All rights reserved.

Change History

Note: The change history is for internal reference only. You must delete the change history when this document is released for external users.

| Date | Version | Description | Revised |
| --- | --- | --- | --- |
| 2023-05-26 | V1.0 | First draft. |  |

- Product Overview

The RG-RSR20-XA series multi-service access router is the next-generation fixed access router innovatively launched by Ruijie Network to cope with enterprise network cloud architecture and cloud service demands.

It integrates routing, switching, security, and other access network applications, while comprehensively improving the performance, port density, rate, reliability, and other aspects. It can provide a low-cost integrated solution for enterprise edge networks, satisfying diversified application needs and trends of future service expansion.

It comes with a built-in stateful firewall function, providing excellent capabilities to defend against traffic attacks. It also boasts a wide range of software features, including Multiprotocol Label Switching (MPLS), Virtual Private Network (VPN), multicast, IPv6, and Network Address Translation (NAT). Additionally, it offers rich backup solutions and QoS/HQoS features, without the need for separate software licenses or service cards, saving investment costs.

It supports the next-generation expansion module HSIC card. The HSIC card supports up to 10 Gbps bus bandwidth, and various types of HSIC cards are available to support more combined applications. The HSIC module is hot-swappable, which is only available on a high-end router. The line card can be added or replaced online without affecting forwarding of other services. The router also supports power module redundancy, which enhances device reliability.

The RG-RSR20-XA series is available in three models: RG-RSR20-XA-24, RG-RSR20-XA-36, and RG-RSR20-XA-54.

It is suitable for small- and medium-sized branch scenarios of finance, government, and enterprise customers, providing an ideal choice for the low-cost and multi-service access needs of this scenario.

Product Appearance

RG-RSR20-XA-24

Front View of the RG-RSR20-XA-24

RG-RSR20-XA-36

Front View of the RG-RSR20-XA-36

RG-RSR20-XA-54

Front View of the RG-RSR20-XA-54

Rear View of the RG-RSR20-XA-54

Product Highlights

Full-Featured Service Router in Diverse Scenarios

Routing, switching, security (IPsec), egress (NAT), and 4G/5G

E1/POS/GE/10GE rates

Carrier-Class Reliability

BOOT dual planes, ensuring stable operation with 99.999% uptime

Redundancy of key components such as fan modules and power modules

Product Features

Multi-service Integration and All-in-One

Advanced Hardware Architecture

Multi-core processor and distributed hardware architecture, meeting diversified network access needs

Distributed hardware architecture that separates the control plane from the forwarding plane, and delivers excellent forwarding performance and reliability

Powerful Routing Capabilities

IPv4/IPv6 static routing and dynamic routing protocols including the Open Shortest Path First (OSPF), Routing Information Protocol (RIP), Intermediate System to Intermediate System (IS-IS), and Border Gateway Protocol (BGP), adapting to different networking requirements of various industries

Policy-based routing (PBR) used to flexibly control and schedule network traffic, satisfying routing requirements of finance, government, and enterprise networks

High-performance flow table, fast forwarding technologies, and functions such as Graceful Restart (GR), Fast Reroute (FRR), and Bidirectional Forwarding Detection (BFD), realizing fast convergence of traditional routing protocols on large-sized networks

Comprehensive Switching Characteristics

Layer 2 Ethernet interfaces extended to integrate functions of a switch and a router, reducing the investment in network construction and maintenance costs

Layer 2 Ethernet interfaces that support complete switching features and provide 1000M line-speed forwarding

Rich VPN Features

Internet Protocol Security (IPsec), Generic Routing Encapsulation (GRE), Dynamic Multipoint Virtual Private Network (DMVPN), and other VPN applications

Multiprotocol Label Switching (MPLS) L2VPN/L3VPN applications

Layer 2 Tunneling Protocol (L2TP) or Point to Point Tunneling Protocol (PPTP) for Virtual Private Dialup Network (VPDN) applications, and L2TP/PPTP pass-through for Network Address Translation (NAT) applications

Various Security Features

Built-in encryption engine, substantially improving encryption and decryption performance

Rich security features, including the firewall, IPsec VPN, Secure Shell (SSH), intrusion defense, DDoS defense, attack defense, anti-ARP spoofing, DHCP snooping, and digital certificate

Authentication, Authorization, and Accounting (AAA), and RADIUS and TACACS+ authentication

802.1X authentication, MAC address authentication, web authentication, and other access authentication modes

All-round MPLS Functions

All-round MPLS protocols including MPLS L2VPN and L3VPN, providing a high-performance, secure, and hierarchical MPLS VPN solution

Distributed multicast VPN, enabling high-performance multicast services on an MPLS VPN network to cope with multicast services such as video conferencing and distance learning

MPLS Label Distribution Protocol (LDP) FRR for fast label switched path (LSP) switching

Complete IPv6 Solutions

IPv4/IPv6 dual stack, and multiple IPv6 routing technologies such as static routing, Equal-cost Multi-path Routing (ECMP), PBR, OSPFv3, RIPng, BGP4+, and IS-ISv6, supporting future deployment of large-scale IPv6 networks

Rich IPv4-to-IPv6 transition solutions, such as dual-stack, manual tunnels, ISATAP tunnels, and 6to4 tunnels, allowing transition from an IPv4 network to an IPv6 network

A variety of IPv6-related technologies such as address auto-configuration, Internet Control Message Protocol Version 6 (ICMPv6), ICMPv6 redirection, ACL for IPv6, and TCP/UDP for IPv6

Video features and optimization

Diverse multicast functions including Internet Group Management Protocol (IGMP), Protocol Independent Multicast - Dense Mode (PIM-DM), Distance Vector Multicast Routing Protocol (DVMRP), and PIM Sparse Mode (PIM-SM) to meet video application requirements

Sound HQoS/QoS Policies

5-level Hierarchical QoS (HQoS): HQoS enables the router to process data flows on a network based on the combination of user services, users, user groups, and interfaces, to achieve multi-level QoS. Underlying user service-level QoS ensures the SLA between multiple service flows of a single user, user-level QoS ensures the SLA between multiple users within a group, user group-level and interface-level QoS achieve SLAs between user groups and on an interface respectively. HQoS realizes E2E service quality guarantee in a centralized manner. It also improves the QoS accuracy on a data aggregation device and enhances the quality of services for network-wide users.

QoS traffic queue scheduling policies such as Priority Queuing (PQ), Custom Queuing (CQ), First In First Out (FIFO), Weighted Fair Queuing (WFQ), Class-based WFQ (CBWFQ) , Low Latency Queuing (LLQ), and Real-time Transport Protocol Priority Queuing (RTPQ), and HQoS multi-level queue scheduling technologies such as flow queue (FQ), user queue (UQ), user group queue (GQ), virtual output queue (VoQ), destination interface/destination device class queue (CQ), and low priority queue (LPQ)

Ease of Use and Management

Command line interface (CLI), web, Simple Network Management Protocol (SNMP), CPE WAN Management Protocol (CWMP), and other modes

Onboarding through Zero Touch Provisioning (ZTP), and onsite configuration import and software upgrade through the USB flash drive and wizard, avoiding complex CLI operations, simplifying device deployment, and greatly improving deployment efficiency

High Reliability

BFD, BFD for static routes, and BFD for RIP/OSPF/BGP/IS-IS, achieving FRR and millisecond-level switching

Hot swapping of line cards: Services are not affected when line cards are added or replaced online, which improves the device reliability.

The RG-RSR20-XA series supports dual power module redundancy to enhance reliability.

Product Specifications

Hardware Specifications

| Hardware Specifications | RG-RSR20-XA-24 | RG-RSR20-XA-36 | RG-RSR20-XA-54 |
| --- | --- | --- | --- |
| Interface Specifications |  |  |  |
| Fixed WAN port | 4 x 10GE optical ports 4 x 1GE optical ports 4 x 1GE electrical ports | 4 x 10GE optical ports 4 x 1GE optical ports 4 x 1GE electrical ports | 2 x 10GE optical ports 4 x 1GE electrical ports |
| Fixed LAN port (LAN/WAN port switching) | 12 x 1GE electrical ports | 24 x 1GE electrical ports | 48 x 1GE electrical ports |
| Power modules | 2 (1+1 redundancy) | 2 (1+1 redundancy) | 2 (1+1 redundancy) |
| Fixed management ports | 1 x console port and 1 x USB 2.0 port |  |  |
| Expansion slot | 4 |  |  |
| System Specifications |  |  |  |
| Packet forwarding rate | 9 Mpps to 40 Mpps |  |  |
| System switching capacity | 120 Gbps |  |  |
| Number of MAC addresses | Number of global MAC addresses: 10,000 Number of static MAC addresses: 4,000 |  |  |
| ARP table size | 24,000 |  |  |
| Number of IPv4 unicast routes | 100,000 |  |  |
| Number of IPv4 multicast routes | 2,048 |  |  |
| Number of IPv6 multicast routes | 1,024 |  |  |
| Number of ACEs | Ingress: 4,000 Egress: 4,000 |  |  |
| Dimensions and Weight |  |  |  |
| Dimensions (W x D x H) | 440 mm x 420 mm x 44 mm (17.32 inch. x 16.56 in. x 1.73 in.), 1 RU |  |  |
| Weight (Full configuration) | < 7.8 kg (17.20 lbs) (excluding packaging) |  |  |
| CPU and Storage |  |  |  |
| CPU | FT2000_V2 |  |  |
| Storage | 4 GB DDR4 8 GB eMMC |  |  |
| Power and Consumption |  |  |  |
| Maximum power consumption | < 150 W |  |  |
| Maximum output power | RG-PA150IB-F: 150 W @ 10 A power cord |  |  |
| Rated input voltage | RG-PA150IB-F: 100 V AC to 240 V AC, 50 Hz to 60 Hz |  |  |
| Maximum input voltage | RG-PA150IB-F: 90 V AC to 264 V AC, 50 Hz to 60 Hz |  |  |
| Environment and Reliability |  |  |  |
| MTBF | > 100,000 hours |  |  |
| Operating temperature | 0°C to 50°C (32°F to 122°F) |  |  |
| Storage temperature | –40°C to +70°C (–40°F to +158°F) |  |  |
| Operating humidity | 10% to 90% RH (non-condensing) |  |  |
| Storage humidity | 5% to 95% RH (non-condensing) |  |  |
| Interface surge protection | 6 kV |  |  |
| Operating altitude | 0–3000 m (0–9,842.52 ft.) |  |  |

Software Specifications

| RG-RSR20-XA Series |  |
| --- | --- |
| Model | Description |
| Ethernet switching | Jumbo frame length: 1,578 bytes |
|  | Number of created VLANs: 4,088 |
|  | Voice VLAN |
|  | MAC address-based and interface-based VLAN assignment |
|  | STP (IEEE 802.1.d), RSTP (IEEE 802.1w), and MSTP (IEEE 802.1s) |
|  | LLDP/LLDP-MED |
| IP service | DHCP server, DHCP relay, DHCP client, and DHCP snooping |
|  | Static and dynamic NAT, static and dynamic NAPT, and VPN NAT |
|  | DNS |
|  | DHCPv6 client, DHCPv6 relay, and DHCPv6 snooping |
|  | Neighbor Discovery (ND) |
|  | IPv6 GRE tunnel |
| IP routing | IPv4/IPv6 static routing |
|  | RIP v1/v2 and RIPng |
|  | OSPFv2 and OSPFv3 |
|  | GR |
|  | IS-ISv4 and IS-ISv6 |
|  | BGP and BGP4+ |
|  | Routing policies |
|  | IPv4/IPv6 VRF |
|  | IPv4 and IPv6 PBR |
| Multicast | IGMPv1/v2/v3 |
|  | Static multicast routing |
|  | PIM-DM, PIM-SM, PIM-SSM, and other multicast routing protocols |
|  | PIM-SSM for IPv4 and IPv6 |
|  | MSDP for inter-domain multicast |
|  | MLDv1/v2 |
|  | PIM-SMv6 |
| MPLS | MPLS forwarding Two MPLS VPN modes: inter-AS Option B and Option C MPLS L3VPN |
|  | 6PE/6VPE interconnection with IPv4/IPv6 MPLS backbone network |
|  | MPLS MIB (RFC 1273, 4265, 4382) |
| ACL and QoS | Standard IP ACLs |
|  | Extended IP ACLs |
|  | Extended MAC ACLs |
|  | Expert-level ACLs (ACLs based on source and destination IP addresses and ports, and specified protocol) |
|  | IPv6 ACLs |
|  | HQoS (traffic shaping on interfaces) |
| Security | Multiple AAA modes |
|  | RADIUS authentication and authorization |
|  | TACACS+ |
|  | IEEE802.1X authentication, MAC address bypass (MAB) authentication, and interface-based and MAC address-based 802.1X authentication |
|  | MAC address allowlist, and bindings of VLAN IDs, interface numbers, and MAC addresses |
|  | SSHv1 and SSHv2 |
|  | ICMP (interface discards ICMP traffic of which the rate exceeds the threshold) |
|  | Port security |
|  | L2TP, IPsec, and GRE uRPF Login authentication and password security |
|  | CPP |
|  | Strict and loose RPF uRPF ignoring default routes |
| Reliability | RLDP, Layer 2 link connectivity detection, unidirectional link detection, and VLAN-based loop control |
|  | DLDP |
|  | IPv4 VRRP v2/v3 and IPv6 VRRP |
|  | ECMP |
|  | BFD |
|  | GR for OSPF/IS-IS/BGP BFD for VRRP, OSPF, BGP4, IS-IS, IS-ISv6, MPLS, and static routing |
| SDN technology | NETCONF |
| NMS and maintenance | SPAN and ERSPAN |
|  | NTP and SNTP |
|  | NTP server and NTP client |
|  | FTP and TFTP |
|  | SNMP v1/v2/v3 |
|  | System operation logging |
|  | Port mirroring |
|  | Command line through console, AUX Modem, Telnet, and SSH |
|  | RMON (1, 2, 3, 9) |
|  | CWMP |
|  | gRPC |

Note: The item marked with the asterisk (*) will be available in the future.

Protocol Compliance

| RG-RSR20-XA Series |  |
| --- | --- |
| Organization | Standards and Protocol |
| IETF | RFC 1058 Routing Information Protocol (RIP) RFC 1157 A Simple Network Management Protocol (SNMP) RFC 1305 Network Time Protocol Version 3 (NTP) RFC 1349 Internet Protocol (IP) RFC 1350 TFTP Protocol (revision 2) RFC 1519 CIDR RFC 1583 OSPF Version 2 RFC 1591 Domain Name System Structure and Delegation RFC 1643 Ethernet Interface MIB RFC 1757 Remote Network Monitoring (RMON) RFC 1812 Requirements for IP Version 4 Router RFC 1901 Introduction to Community-based SNMPv2 RFC 1902-1907 SNMP v2 RFC 1918 Address Allocation for Private Internet RFC 1981 Path MTU Discovery for IP version 6 RFC 1997 BGP Communities Attribute RFC 2131 Dynamic Host Configuration Protocol (DHCP) RFC 2132 DHCP Options and BOOTP Vendor Extensions RFC 2236 IGMP RFC 2328 OSPF Version 2 RFC 2385 Protection of BGP Sessions via the TCP MD5 Signature Option RFC 2439 BGP Route Flap Damping RFC 2460 Internet Protocol, Version 6 (IPv6) RFC 2461 Neighbor Discovery for IP Version 6 (IPv6) RFC 2462 IPv6 Stateless Address Auto configuration RFC 2463 Internet Control Message Protocol for IPv6 (ICMPv6) RFC 2545 Use of BGP 4 Multiprotocol Extensions for IPv6 Inter Domain Routing RFC 2571 SNMP Management Frameworks RFC 2711 IPv6 Router Alert Option RFC 2787 Definitions of Managed Objects for the Virtual Router Redundancy Protocol RFC 2863 The Interfaces Group MIB RFC 2918 Route Refresh Capability for BGP 4 RFC 2925 Definitions of Managed Objects for Remote Ping, Traceroute, and Lookup Operations (Ping only) RFC 2934 Protocol Independent Multicast MIB for IPv4 RFC 3046 DHCP Option82 RFC 3065 Autonomous System Confederation for BGP RFC 3101 OSPF Not so stubby area option RFC 3137 OSPF Stub Router Advertisement sFlow RFC 3417 (SNMP Transport Mappings) RFC 3418 Management Information Base (MIB) for the Simple Network Management Protocol (SNMP) RFC 3509 Alternative Implementations of OSPF Area Border Routers RFC 3513 IP Version 6 Addressing Architecture RFC 3623 Graceful OSPF Restart RFC 3768 VRRP RFC 3810 Multicast Listener Discovery Version 2 (MLDv2) for IPv6 RFC 3973 PIM Dense Mode RFC 4022 MIB for TCP RFC 4271 A Border Gateway Protocol 4 (BGP 4) RFC 4273 Definitions of Managed Objects for BGP 4 RFC 4360 BGP Extended Communities Attribute RFC 4456 BGP Route Reflection: An Alternative to Full Mesh Internal BGP (IBGP) RFC 4486 Subcodes for BGP Cease Notification Message RFC 4552 Authentication/Confidentiality for OSPFv3 RFC 4724 Graceful Restart Mechanism for BGP RFC 4750 OSPFv2 MIB partial support no SetMIB RFC 4760 Multiprotocol Extensions for BGP 4 RFC 4940 IANA Considerations for OSPF RFC 5065 Autonomous System Confederation for BGP RFC 5187 OSPFv3 Graceful Restart RFC 5340 OSPFv3 for IPv6 RFC 5492 Capabilities Advertisement with BGP 4 RFC 6620 FCFS SAVI RFC 768 User Datagram Protocol (UDP) RFC 783 TFTP Protocol (revision 2) RFC 792 Internet Control Message Protocol (ICMP) RFC 793 Transmission Control Protocol (TCP) RFC 813 Window and Acknowledgement Strategy in TCP RFC 815 IP datagram reassembly algorithms RFC 826 Ethernet Address Resolution Protocol (ARP) RFC 854 Telnet Protocol RFC 959 File Transfer Protocol (FTP) RFC 4251 The Secure Shell (SSH) Protocol RFC 4252 SSHv6 Authentication RFC 4253 SSHv6 Transport Layer RFC 4254 SSHv6 Connection RFC 4291 IP Version 6 Addressing Architecture RFC 4292 IP Forwarding Table MIB RFC 4293 Management Information Base for the Internet Protocol (IP) RFC 4443 ICMPv6 RFC 4861 IPv6 Neighbor Discovery RFC 5424 Syslog Protocol RFC 5798 VRRP RFC 5905 Network Time Protocol Version 4: Protocol and Algorithms RFC 6241 Network Configuration Protocol (NETCONF) |
| IEEE | IEEE 802.2 Logical Link Control IEEE 802.1AB 2005 IEEE 802.1ab Link Layer Discovery Protocol IEEE 802.1ad Provider Bridges IEEE 802.1ax/IEEE802.3ad Link Aggregation IEEE 802.1D Media Access Control (MAC) Bridges IEEE 802.1D Spanning Tree Protocol IEEE 802.1Q Virtual Bridged Local Area NetworksVLAN IEEE 802.1s Multiple Spanning Tree Protocol IEEE 802.1w Rapid Spanning Tree Protocol IEEE 802.1x Port based network access control protocol IEEE Std 802.3ab 1000BASE-T IEEE 802.3ad Link Aggregation Control Protocol (LACP) IEEE Std 802.3ae 10GE WEN/LAN Standard IEEE Std 802.3x Full Duplex and flow control IEEE Std 802.3z Gigabit Ethernet Standard |

Typical Applications

Financial Branch Network Access

Networking:

The RSR77XA series high-end routers are used in the provincial level-1 and level-2 banks, and the RSR20-XA routers are deployed on a branch network to meet multi-service access requirements.

The RG-RSR20-XA series supports MSTP, 4G/5G, E1, Ethernet, and other access modes.

Enterprise Branch Interconnection

Networking:

The RSR30-XA series routers are deployed in the headquarters, and the RSR20-XA series routers are deployed in branches.

The RG-RSR20-XA series supports MSTP, GE, E1, 4G/5G, and other access modes.

It supports IPsec VPN, AAA, digital certificates, and other technologies.

Ordering Guide

Perform the following steps to configure an RG-RSR20-XA:

Select the chassis based on the specific product model.

Select a power module according to power supply requirements of the device. At least one power module is required.

Select line cards as needed.

Select accessories such as optical modules, antennas, cables, and adapters according to interface types.

Models marked with asterisks (*) in the ordering information are available later.

Ordering Information

Chassis

| Model | Description |
| --- | --- |
| RG-RSR20-XA-24 | RG-RSR20-XA-24 chassis, 12 x fixed WAN ports (4 x 10GE optical ports, 4 x 1GE optical ports, and 4 x 1GE electrical ports) and 12 x 1GE electrical LAN ports (LAN/WAN port switching), 4 x Expansion slots, 1 x USB 2.0 port, 1 x console port |
| RG-RSR20-XA-36 | RG-RSR20-XA-36 chassis, 12 x fixed WAN ports (4 x 10GE optical ports, 4 x 1GE optical ports, and 4 x 1GE electrical ports) and 24 x 1GE electrical LAN ports (LAN/WAN port switching), 4 x Expansion slots, 1 x USB 2.0 port, 1 x console port |
| RG-RSR20-XA-54 | RG-RSR20-XA-54 chassis, 6 x fixed WAN ports (2 x 10GE optical ports and 4 x 1GE electrical ports) and 48 x 1GE electrical LAN ports (LAN/WAN port switching), 4 x Expansion slots, 1 x USB 2.0 port, 1 x console port |

Power Modules

| Model | Description |
| --- | --- |
| RG-PA150IB-F | 150 W AC power module |

Line Card

| Model | Description |
| --- | --- |
| HSIC-2E1/CE1 | 2-port E1/CE1 module (the cable needs to be purchased separately) |
| HSIC-4G-LTE | 1-port full-mode 4G and encryption module (the antenna needs to be purchased separately) |
| *HSIC-5G-NR | 1-port full-mode 5G and encryption module (the antenna needs to be purchased separately) |
| *HSIC-1POS-STM1 | 1-port OC-3c/STM-1c POS interface card (the optical module needs to be purchased separately) |

Note: The item marked with the asterisk (*) will be available in the future.

Warranty

For more information about warranty terms and period, contact your local sales agency:

Warranty terms: https://www.ruijienetworks.com/support/servicepolicy

Warranty period: https://www.ruijienetworks.com/support/servicepolicy/Service-Support-Summany/

- Note: The warranty terms are subject to the terms of different countries and distributors.

More Information

For more information about Ruijie Networks, visit the official Ruijie website or contact your local sales agency:

Ruijie Networks official website: https://www.ruijienetworks.com/

Online support: https://www.ruijienetworks.com/support

Hotline support: https://www.ruijienetworks.com/support/hotline

Email support: service_rj@ruijienetworks.com