RG-RSR77-XA Series

Routers

Datasheet

V1.0

Ruijie Networks Co., Ltd

All rights reserved.

Change History

Note: The change history is for internal reference only. You must delete the change history when this document is released for external users.

| Date | Version | Description | Revised |
| --- | --- | --- | --- |
| 2023-05-26 | V1.0 | First draft. |  |

- Product Overview

The RSR77-XA series router is the next-generation high-end distributed router developed by Ruijie Networks for the cloud architecture and cloud service of the enterprise network.

Based on Ruijie-developed RGOS 12.X operating system, the RSR77-XA series multi-service core router features high performance, high reliability, and high security. The RG-RSR77-XA series supports smooth upgrade and capacity expansion, providing strong support for live network services and coping with service development in the next five to ten years.

The chassis uses independent routing switching engines, switch fabric units (SFUs), and line cards, separating management, control, and data planes based on the high-performance multi-core platform and distributed architecture. The RG-RSR77-XA series adopts dual data planes, improving performance and ensuring nonstop services.

Characterized by high availability, high performance, multi-service, and high security, the RG-RSR77-XA series supports enterprise service operation and bearer network construction, enhancing network value and saving network construction cost. With a resilient and scalable architecture, the RG-RSR77-XA series can boost its performance by installing more line cards. What is more, traffic can be load balanced among line cards inside the chassis.

Based on Ruijie-developed RGOS 12.X operating system, the RSR77-XA series provides various features. High-speed forwarding technologies such as the virtual Central Processing Unit (vCPU) and Ruijie Express Forwarding (REF) improve the router's multi-service processing capability. The distributed service line cards of the RG-RSR77-XA series have various types of ports and integrate features including Network Address Translation (NAT), Internet Protocol Flow Information Export (IPFIX), Internet Protocol Version 6 (IPv6), Border Gateway Protocol (BGP), IP Security (IPsec), Multi-Protocol Label Switching Virtual Private Network (MPLS VPN), and Hierarchical Quality of Service (H-QoS), and multicast features.

The RG-RSR77-XA series is available in two models: RG-RSR77XA-08 and RG-RSR77XA-03.

The RG-RSR77-XA series is mainly used as the core or aggregation node on a WAN of the government, finance, carrier, education, or large- and medium-sized enterprise network, egress of a large campus network, and an edge aggregation node of a data center. The RG-RSR77-XA series together with RSR30-XA and RSR20-XA series routers enables one-stop delivery of a WAN solution.

Product Appearance

RG-RSR77-XA-03

Front View of the RG-RSR77-XA-03

RG-RSR77-XA-08

Front View of the RG-RSR77-XA-08

Product Highlights

Multi-service core router, supporting IP, MPLS, SR, IPsec, L2TP, and SRv6, with no limitations on service provisioning

Multi-rate interfaces, including E1/CPOS/POS/GE/10GE/40GE/100GE interfaces, meeting requirements of different network layers

Hardware redundancy to ensure uninterrupted service: Key components such as supervisor modules, switch fabric modules, and power modules use redundancy design, and the RG-RSR77-XA-03 has no switch fabric module and integrates the supervisor module and switch fabric module.

Y-shaped airflow design with power consumption lower than 0.5 W per Gbps, reducing overall energy consumption

Product Features

Comprehensive Architecture Upgrade

Based on a resilient and scalable platform, the chassis of the RG-RSR77-XA series uses Clos non-blocking switching and advanced distributed multi-service processing architecture and provides 100GE and 40GE ports. The chassis uses independent SFUs and separated routing, service, and forwarding engines, separating management, control, and data planes. This further improves service availability.

Modular Compatibility

The RG-RSR77-XA series leverages the independent carrier card design and comes into two modular models: RG-RSR77-XA-08 and RG-RSR77-XA-03. The two models employ consistent hardware and software platforms with completely compatible line cards, adapting to application requirements at different network layers for constantly developing enterprises.

Energy-Saving Design

The RG-RSR77-XA series incorporates the energy-saving design. The chassis supports highly-efficient modular power modules, which improves the power system efficiency and reduces energy consumption of an equipment room.

The chassis adopts heat dissipation by isolating physical partitions. Intelligent fan modules support partition-based temperature control and multi-level speed regulation, saving energy and reducing noise. All these measures greatly reduce energy consumption of air conditioners.

High-performance and Built-in Services

The distributed line cards of the RG-RSR77-XA series have built-in features including IPv6, BGP, IPsec, MPLS VPN, and QoS. When new service features need to be loaded, there is no need to purchase a software license or add a line card, maximizing user investment.

Powerful Route Processing Capability

The RG-RSR77-XA series supports various dynamic routing protocols to apply to different networks. Furthermore, it supports policy-based routing (PBR) to flexibly control and schedule network traffic, coping with routing demands of finance, government, and enterprise customers.

Based on the high-performance flow table and fast forwarding technologies, the RG-RSR77-XA series uses features such as GR, FRR, and BFD to accelerate convergence of traditional routing protocols, achieving rapid convergence on a large-sized network.

All-round MPLS VPN Functions

The RG-RSR77-XA series supports comprehensive MPLS VPN functions. It can function as the provider (P) or provider edge (PE) device, and delivers fully distributed Layer 2 and Layer 3 VPN services. It provides a high-performance, secure, and hierarchical MPLS VPN solution.

The RG-RSR77-XA series supports distributed multicast VPN and can be enabled with high-performance multicast service on the MPLS VPN network, satisfying multicast service requirements of video conference and remote teaching.

The RG-RSR77-XA series provides fault location, MPLS ping, and MPLS traceroute to help users quickly locate failure points.

Full SRv6 Support

SRv6 supported by the RG-RSR77-XA series can be used with the Network Configuration Protocol (NETCONF), Topology-Independent Loop-free Alternate (TI-LFA), inter-AS E2E, and PCEP. The RG-RSR77-XA series supports BGP for SRv6 policies, Generalized SRv6 (G-SRv6), SRv6 OAM, SRv6 Policy, SRv6-BE, SRv6 GR, Non-Stop Routing (NSR) for SRv6, and Seamless Bidirectional Forwarding Detection (SBFD) for SRv6.

Carrier-Class Reliability

Redundancy of key modules: routing engines, SFUs, power modules in N+M mode, dual startup image files, and dual configuration files.

Hot swapping of key components: routing engines, carrier cards, line cards, power modules, and fan modules

Flexible Device Management

The RG-RSR77-XA series supports user-based hierarchical management and password protection, and applies the multi-core CPU technology to separate the control plane from the forwarding plane, allowing device management in any environment. It authenticates login users and grants users of different levels with different administrative permissions, ensuring device security.

Product Specifications

Hardware Specifications

| Hardware Specifications | RG-RSR77-XA-03 | RG-RSR77-XA-08 |
| --- | --- | --- |
| Interface Specifications |  |  |
| Power module | 4 (N+N redundancy) |  |
| Fan module | 1 | 2 |
| Fixed management port | 1 x console port, 1 x MGMT port, and 1 x USB port | 1 x console port, 1 x MGMT port, and 1 x USB port |
| Supervisor module slot | 2 | 2 |
| Switch fabric module | 2 x integrated switch fabric modules | 4 x independent switch fabric modules |
| Service module slot | 3 | 8 |
| Line card slot | 12 | 32 |
| System Specifications |  |  |
| Packet forwarding rate | 25,000 Mpps | 35,000 Mpps |
| System Switching capacity | 150 Tbps | 180 Tbps |
| ARP table size | 40,000 | 40,000 |
| Number of IPv4 unicast routes | 500,000 | 500,000 |
| Number of IPv4 multicast routes | 2,048 | 2,048 |
| Number of IPv6 unicast routes | 1,000,000 | 1,000,000 |
| Number of IPv6 multicast routes | 1,024 | 1,024 |
| Number of ACEs | Ingress: 16,000 Egress: 16,000 | Ingress: 16,000 Egress: 16,000 |
| Dimensions and Weight |  |  |
| Dimensions (W x D x H) | 442 mm x 560 mm x 220 mm (17.40 in. x 22.05 in. x 8.66 in.), 5 RU | 442 mm x 560 mm x 442 mm (17.40 in. x 22.05 in. x 17.40 in.), 10 RU |
| Weight (empty chassis and fan modules) | 12.5 kg (27.56 lbs) | 43.55 kg (96.01 lbs) |
| CPU and Storage |  |  |
| CPU | FT2000/D2000 | FT2000/D2000 |
| Storage | Flash: 16 GB SDRAM: 8 GB | Flash: 16 GB SDRAM: 8 GB |
| Power and Consumption |  |  |
| Maximum power consumption | 40 W | 60 W |
| Maximum output power | RG-PA460-RI: 90 V AC to 290 V AC, 460 W | RG-PA1600I: 90 V AC to 180 V AC, 200 W 180 V AC to 264 V AC, 1600 W HDC: 192 V DC to 216 V DC, 1200 W 216 V DC to 290 V DC, 1600 W RG-PD600I: 600 W |
| Rated input voltage | AC: 100 V AC to 120 V AC, 200 V AC to 240 V AC, 50 Hz to 60Hz | AC: 100 V AC to 120 V AC, 200 V AC to 240 V AC, 50 Hz to 60 Hz DC: –48 V DC |
| Maximum input voltage | AC: 90 V AC to 290 V AC, 45 Hz to 65 Hz | AC: 90 V AC to 264 V AC, 47 Hz to 63 Hz DC: –40 V DC to –75 V DC |
| Environment and Reliability |  |  |
| MTBF | 216,000 hours | 342,000 hours |
| Primary airflow | Fans drawing air for forced convection | Fans drawing air for forced convection |
| Operating temperature | 0°C to 45°C (32°F to 113°F) |  |
| Storage temperature | –40°C to +70°C (–40°F to +158°F) |  |
| Operating humidity | 10% to 90% RH (non-condensing) |  |
| Operating noise | Ambient temperature ≤ 70 dB Maximum noise < 77 dB | 35°C (95°F): 60 dB 50°C (122°F): 75 dB |

Software Specifications

| RG-RSR77-XA Series |  |
| --- | --- |
| Model | Description |
| Ethernet switching | Jumbo frame length: 1,578 bytes |
|  | 802.3az EEE |
|  | LLDP/LLDP-MED |
| IP service | Static and dynamic ARP |
|  | DHCP server, DHCP relay, and DHCP client |
|  | FTP, TFTP, FTPv6, and TFTPv6 |
|  | NTP, SNTP, and NTPv6 |
|  | NAT |
|  | Strict and loose RPF uRPF ignoring default routes |
|  | ND |
|  | FTP, TFTP, FTPv6, and TFTPv6 |
|  | DNS |
|  | IPv4/IPv6 VRF |
|  | DHCP server, DHCPv6 client, and DHCPv6 relay |
|  | ND and ND snooping |
|  | IPv6 GRE tunnel |
| IP routing | IPv4 and IPv6 static routing, and static blackhole routes |
|  | RIP v1/v2 and RIPng |
|  | OSPFv2 and OSPFv3 |
|  | IS-ISv4 and IS-ISv6 |
|  | BGP and BGP4+ |
|  | Routing policies |
|  | BGP4, BGP4+, and MP-BGP |
|  | IPv4 VRRP v2/v3 and IPv6 VRRP v3 |
|  | IPv4 and IPv6 PBR |
| Multicast | IGMP v1/v2/v3 |
|  | Static multicast routing |
|  | PIM-DM, PIM-SM, PIM-SSM, PIM-SSMv6, PIM-SMv6, and other multicast routing protocols |
|  | MSDP for inter-domain multicast |
|  | MLDv1/v2 |
| MPLS/SR | Basic MPLS functions |
|  | MPLS forwarding |
|  | 6PE/6VPE interconnection with IPv4/IPv6 MPLS backbone network |
|  | MPLS MIB (RFC 1273, 4265, 4382) |
|  | L3VPN |
|  | LSP |
|  | LDP |
| ACL and QoS | Standard IP ACLs |
|  | Extended IP ACLs |
|  | Extended MAC ACLs |
|  | IPv6 ACLs |
|  | Applying ACLs to interfaces |
|  | MPLS QoS |
|  | HQoS |
| Security | Multiple AAA modes |
|  | RADIUS authentication, authorization, and accounting |
|  | TACACS+ |
|  | Attack defense |
|  | Password security |
|  | RNFP |
|  | CPP |
|  | FPM |
|  | PKI |
|  | IPsec VPN |
|  | VPDN |
|  | SSHv1 and SSHv2 |
|  | Global IP-MAC binding |
|  | Login authentication and password security |
| Reliability | BFD |
|  | GR |
|  | DLDP |
|  | ECMP |
|  | NSR |
|  | Hot swapping |
| NMS and maintenance | SPAN and ERSPAN |
|  | SNMP v1/v2/v3 |
|  | System operation logging |
|  | IPFIX |
|  | Command line configuration through console, AUX Modem, Telnet, and SSH |
|  | RMON (1, 2, 3, 9) |
|  | NETCONF |
|  | CWMP |
|  | gRPC |

Note: The item marked with the asterisk (*) will be available in the future.

Protocol Compliance

| RG-RSR77-XA Series |  |
| --- | --- |
| Organization | Standards and Protocol |
| IETF | RFC 1157 A Simple Network Management Protocol (SNMP) RFC 1305 Network Time Protocol Version 3 (NTP) RFC 1349 Internet Protocol (IP) RFC 1350 TFTP Protocol (revision 2) RFC 1519 CIDR RFC 1591 Domain Name System Structure and Delegation RFC 1643 Ethernet Interface MIB RFC 1757 Remote Network Monitoring (RMON) RFC 1812 Requirements for IP Version 4 Router RFC 1901 Introduction to Community-based SNMPv2 RFC 1902-1907 SNMP v2 RFC 1918 Address Allocation for Private Internet RFC 2131 Dynamic Host Configuration Protocol (DHCP) RFC 2132 DHCP Options and BOOTP Vendor Extensions RFC 2571 SNMP Management Frameworks RFC 2863 The Interfaces Group MIB RFC 2925 Definitions of Managed Objects for Remote Ping, Traceroute, and Lookup Operations (Ping only) RFC 3046 DHCP Option82 RFC 3417 (SNMP Transport Mappings) RFC 3418 Management Information Base (MIB) for the Simple Network Management Protocol (SNMP) RFC 4022 MIB for TCP RFC 768 User Datagram Protocol (UDP) RFC 783 TFTP Protocol (revision 2) RFC 792 Internet Control Message Protocol (ICMP) RFC 793 Transmission Control Protocol (TCP) RFC 813 Window and Acknowledgement Strategy in TCP RFC 815 IP datagram reassembly algorithms RFC 826 Ethernet Address Resolution Protocol (ARP) RFC 854 Telnet Protocol RFC 959 File Transfer Protocol (FTP) RFC 1058 Routing Information Protocol (RIP) RFC 1583 OSPF Version 2 RFC 1981 Path MTU Discovery for IP version 6 RFC 1997 BGP Communities Attribute RFC 2236 IGMP RFC 2328 OSPF Version 2 RFC 2385 Protection of BGP Sessions via the TCP MD5 Signature Option RFC 2439 BGP Route Flap Damping RFC 2460 Internet Protocol, Version 6 (IPv6) RFC 2461 Neighbor Discovery for IP Version 6 (IPv6) RFC 2462 IPv6 Stateless Address Auto configuration RFC 2463 Internet Control Message Protocol for IPv6 (ICMPv6) RFC 2545 Use of BGP 4 Multiprotocol Extensions for IPv6 Inter Domain Routing RFC 2711 IPv6 Router Alert Option RFC 2787 Definitions of Managed Objects for the Virtual Router Redundancy Protocol RFC 2918 Route Refresh Capability for BGP 4 RFC 2934 Protocol Independent Multicast MIB for IPv4 RFC 3065 Autonomous System Confederation for BGP RFC 3101 OSPF Not so stubby area option RFC 3137 OSPF Stub Router Advertisement sFlow RFC 3509 Alternative Implementations of OSPF Area Border Routers RFC 3513 IP Version 6 Addressing Architecture RFC 3623 Graceful OSPF Restart RFC 3768 VRRP RFC 3810 Multicast Listener Discovery Version 2 (MLDv2) for IPv6 RFC 3973 PIM Dense Mode RFC 4271 A Border Gateway Protocol 4 (BGP 4) RFC 4273 Definitions of Managed Objects for BGP 4 RFC 4360 BGP Extended Communities Attribute RFC 4456 BGP Route Reflection: An Alternative to Full Mesh Internal BGP (IBGP) RFC 4486 Subcodes for BGP Cease Notification Message RFC 4552 Authentication/Confidentiality for OSPFv3 RFC 4724 Graceful Restart Mechanism for BGP RFC 4750 OSPFv2 MIB partial support no SetMIB RFC 4760 Multiprotocol Extensions for BGP 4 RFC 4940 IANA Considerations for OSPF RFC 5065 Autonomous System Confederation for BGP RFC 5187 OSPFv3 Graceful Restart RFC 5340 OSPFv3 for IPv6 RFC 5492 Capabilities Advertisement with BGP 4 |
| IEEE | IEEE 802.2 Logical Link Control IEEE 802.1ab Link Layer Discovery Protocol IEEE 802.1ad Provider Bridges IEEE 802.1ax/IEEE802.3ad Link Aggregation IEEE 802.1D Media Access Control (MAC) Bridges IEEE 802.1Q Virtual Bridged Local Area Networks (VLAN) IEEE 802.3ad Link Aggregation Control Protocol (LACP) IEEE Std 802.3x Full Duplex and flow control |

Typical Applications

E-government Network

Characteristics:

The RG-RSR77-XA series core router supports the MPLS VPN solution for the government industry. It caters to application requirements of e-government intranet and internet.

Financial Backbone Network

Characteristics:

The RG-RSR77-XA series core router can aggregate services of a large number of branches and delivers high performance even in the case of multi-service. It copes with service requirements of backbone networks of banking, insurance, postal savings bank, and other financial subsectors.

Ordering Guide

Perform the following steps to configure an RSR77-XA series router:

Select the chassis and supervisor engine based on the specific product model.

Select switch fabric modules based on service requirements.

Select power modules based on power supply requirements. At least one power module is required.

Select line cards and function modules based on service requirements.

Ordering Information

Chassis and Supervisor Engines

| Model | Description |
| --- | --- |
| RG-RSR77-XA-03 | RG-RSR77-XA-03 chassis |
| RG-RSR77-XA-08 | RG-RSR77-XA-08 chassis |
| RSR77-XA-03-CM | RG-RSR77-XA-03 routing switching engine |
| RSR77-XA-08-CM | RG-RSR77-XA-08 routing engine |
| RSR77-XA-08-DSF | RG-RSR77-XA-08 switch fabric module (40 Gbps supported by a single switch fabric module) |
| RSR77-XA-08-DSF II | RG-RSR77-XA-08 switch fabric module (200 Gbps supported by a single switch fabric module) |

Power Modules

| Model | Description |
| --- | --- |
| RG-PA1600I | RG-RSR77-XA-08, 1600 W AC power module |
| RG-PA460-RI | RG-RSR77-XA-03, 460 W AC power module |
| RG-PD600I | RG-RSR77-XA-08, 600 W DC power module |

SIP Carrier Card

| Model | Description |
| --- | --- |
| RSR77-XA-SIP3 | NM/DNM carrier card (four NM modules or two DNM modules supported by each RSR77-XA-SIP3) |

NM Line Card

| Model | Description |
| --- | --- |
| NM-2XS-KA | 2-port 10GE Ethernet interface module, requiring additional SFP+ modules |
| NM-4XS-KB | 4-port 10GE Ethernet interface module, requiring additional SFP+ modules |
| NM-8SFP-G | 8-port 1GE Ethernet optical interface module, requiring additional SFP modules |
| NM-8GT-G | 8-port 1GE Ethernet electrical interface module |
| NM-8E1/CE1 | 8-port E1/CE1 interface module, requiring adapters and adapter cables |

Note: The item marked with the asterisk (*) will be available in the future.

Warranty

For more information about warranty terms and period, contact your local sales agency:

Warranty terms: https://www.ruijienetworks.com/support/servicepolicy

Warranty period: https://www.ruijienetworks.com/support/servicepolicy/Service-Support-Summany/

- Note: The warranty terms are subject to the terms of different countries and distributors.

More Information

For more information about Ruijie Networks, visit the official Ruijie website or contact your local sales agency:

Ruijie Networks official website: https://www.ruijienetworks.com/

Online support: https://www.ruijienetworks.com/support

Hotline support: https://www.ruijienetworks.com/support/hotline

Email support: service_rj@ruijienetworks.com